The Design, Construction and Monitoring of a Complex Urban Excavation in Stiff Oxford Clay

Response to Reviewer Comments

Brian B. Sheil, Zhandos Orazalin, Ganga Prakhya and Barbara Rossi

Reviewer 1

Reviewer’s comment: 1. The paper cited in Line 25 on Page 3 by Cheng et al (2021) is missing in the reference section.

Response: We thank the reviewer for spotting this. The citation has been added.

Reviewer’s comment: 2. In Line 61 on Page 4, N-values should be blow counts per 0.3m and not 0.15m.

Response: The associated text has been updated.

Reviewer’s comment: 3. The authors may like to report on the design soil parameters (density, stiffness, shear strength, permeability, in situ stresses etc. used in PLAXIS.

Response: Owing to the current length of the manuscript and the word limit for this journal, details of the PLAXIS model has not been included. However, the results of the site investigation have been included.

Reviewer’s comment: 4. The most interesting aspect of the paper is the adoption of the observational approach method. The authors should report clearly at which stage of the construction activities (see Table 1 on Page 12), the construction/design team decided to switch over the design from the more conservative design to the less conservative design viz. the decision to omit some props. 5. The authors should clearly state the criteria for the switch over and the rationale behind the decision. 6. One possible approach was proposed by Pappin et al (2005). A designer may do two designs at the outset as recommended in CIRIA Guide C580: one based on Approach A i.e. “Moderately Conservative” design parameters (with more props) and one based on Approach B i.e. “Most Probable” parameters (with less props). Construction may start with Approach A. The designer should review the design based on the actual site monitoring data. If the data suggest Approach B is more realistic, a decision could then be made to switch the design from Approach A to Approach B.

Response: We thank the reviewer for this helpful suggestion. We have an included an additional section “Application of Observational Method” to address these commands.

Reviewer’s comment: 7. The authors may like to elaborate more on Figure 7(a) and provide more details in the text from Line 112 to Line 115 on Page 6.

Response: This is a sensible suggestion. The section “Real-time monitoring platform” has been extended to include the additional information recommended.

Reviewer’s comment: 8. It would appear the building movements reported in Figure 12 were only those measured during basement excavation i.e. after completion of all secant and contiguous piles on 01/09/2021. This is not reasonable.

Response: That figure (now Figure 14 in the revised version) has been updated to include a longer time history of these movements.

Reviewer’s comment: 9. While the reviewer agrees with the authors that after the secant and contiguous piles were completely installed (wished-in-place), they should be very robust and stiff, but the readers would be more interested in the building movements during the installation of the piles. It is well known that ground will move in the vicinity of pile construction. In particular, the ground movements of SH-ML1, SH-ML2 and SH-ML3 during the installation of the row of secant piles abutting Savile House.

Response: We acknowledge the importance of considering ground movements during pile installation, especially in close proximity to existing structures. However, we are pleased to report that no damage occurred to the Savile House during the installation process. This can be attributed to several factors, including the stiff nature of the Oxford Clay and the implementation of best practices during piling operations.

In response to the reviewer’s suggestion, we have included additional data on ground movements (representing an average of the measurements from monitoring points SH-ML1, SH-ML2, and SH-ML3), during the installation of the secant piles abutting the Savile House (see new Figure 14(a)). This additional information will provide readers with a more comprehensive understanding of the construction process and its impact on the surrounding structures.

Reviewer 2

Reviewer’s comment: The paper contains some interesting monitoring data for a significant excavation in Oxford Clay, for which there are few (if any) other published examples. However, I’m not sure that the paper does much more than report the monitoring and behavior of the excavation during construction: it does not say that much about the observational method, curved wall/corner effects (see further comments below), or consider or back-analyse the wall displacements relative to measured soil stiffnesses, the original design calculations, or the established literature on basements in stiff clays. As such, it is a concise and useful record of wall and soil movements in Oxford Clay (noting that heave is infrequently measured), and for that reason it should be published. It feels to me like detail of the design approach and more detailed comment to/back-analysis of the geotechnical performance would give greater guidance to others constructing in Oxford Clay (on wall movements, and corroboration of bulk stiffness parameters for the clay), curved basement construction (did the curved ends avoid the need for propping within these parts of the basement?) and the observational method. This seems like a missed opportunity, but I accept that the authors may not wish to include any of this in the paper.

Response: We thank the reviewer for these constructive comments. A “Structural Design” section discussing the curved multi-level capping beam and the “arching” effect has now been added to the revised manuscript. A new section “Application of Observational Method” has also been added to elaborate on the progressive design modification used in this project. We hope these modifications achieve the additional contribution recommended by the reviewer.

Reviewer’s comment: Line 26. CIRIA guide C760 on embedded retaining wall design also brings together a lot of case study information from a range of sources to provide guidance on e.g. wall and ground displacements. Lines 27-32: there is of course a lot of case observations from basement structures/walls in the London Clay, and a few in other UK stiff clays/mudrocks, many listed in C760.

Response: This is a very good suggestion; a reference to CIRIA guide C760 has now been added.

Reviewer’s comment: Lines 82 and 83. I failed to understand the difference between the ‘double-level basement raft’ and the ‘base slab’. Please can the authors make this clear?

Response: This sentence was indeed confusing because they are one and the same. This sentence has been clarified in our revised submission (now lines 107 & 108).

Reviewer’s comment: Lines 88 to 93. Noting that the wall is not fixed at the top or the bottom, what is the datum point for inclinometer measurements? The plots in Figure 9(a) may have been taken as laterally fixed at the base, but what is used to determine the lateral movement into the excavation of the other plots for the propped walls – was the top of the wall surveyed, or is a different assumption made? Ditto for the extensometer measurements – what is the point of fixity? It is important to state these so that the results can be fully understood.

Response: The reviewer raises an important point. The bottom of the inclinometer casing served as a stable reference (datum) point; the original plots did not show the full depth of the inclinometers and these have now been updated. The validity of this datum was verified on site using regular surveys of the top of the wall. For the magnetic extensometers, the top capping beam level was always surveyed, and the reported taped vertical distances to centre of magnets were relative to the centre of the base magnet. We have added additional sentences to clarify these points (see lines 194-196 and 206-207).

Reviewer’s comment: Line 99: ‘elastic section modulus’. Do you mean ‘second moment of area’?

Response: Correct – thank you for spotting this type. Z has been replaced with I to indicate that it is indeed the second moment of cross-sectional area (EI is flexural rigidity).

Reviewer’s comment: Line 102. The prop load data in Figure 6 look like they include the effects of temperature, which are causing the diurnal spikes in prop load (and possibly a longer seasonal influence on the load in the props). Presumably the vibrating wire strain gauges had the same coefficient of thermal expansion as the prop and so no adjustment needed to be made to gauge readings for temperature?

Response: Again, another excellent point. This is entirely correct.

Reviewer’s comment: Line 129. By ‘consolidation’ I assume you mean the gradual recovery of pore water pressures after undrained unloading of the soil during excavation? It may be better to spell that out.

Response: Yes. This has now been spelled out as “the dissipation of excess pore water pressures” in our revised manuscript (see line 190).

Reviewer’s comment: Line 158. ‘the prop removal process’. The removal of other props?

Response: Also correct – we have clarified this sentence (now line 222 in revised paper).

Reviewer’s comment: Lines 171 to 183. Conclusions section. This section introduces several new points that I think would be better made earlier in the paper.

Response: Good point - addressed.

Reviewer’s comment: Line 176. The observational method enabled the site team to emit a level of temporary steel props…’ The main body of the paper contains a high level overview of the approach but is very light on detail in relation to the case history (in fact, I don’t think it explains that a propping level was saved) – I think it would be helpful to explain what propping was saved, and what enabled it to be saved- presumably wall movements remaining within limit?

Response: In retrospect, the original paper was indeed light on detail regarding this aspect. We have addressed this comment by introducing a new section entitled “Application of Observational Method”.

Reviewer’s comment: Line 179. The carbon and other (time?) savings would also be better discussed in the main text of the paper.

Response: Corrected.

Reviewer’s comment: Line 180 ‘Measured wall displacements were also well-represented by design values reported for stiff London Clay …’. I think this should also come earlier, and could helpfully be expanded on a little.

Response: Discussion of stiff London Clay has been added to the introduction. An explicit comparison between the present results and historical London Clay measurements (Fernie & Suckling 1996) has also been included in lines 185-186. Additional references have also been added:

- Gasparre, A., Nishimura, S., Coop, M.R. and Jardine, R.J., 2011. The influence of structure on the behaviour of London Clay. In Stiff Sedimentary Clays: Genesis and Engineering Behaviour: Géotechnique Symposium in Print 2007 (pp. 67-79). Thomas Telford Ltd.

- Hight, D.W., Gasparre, A., Nishimura, S., Jardine, R.J., Coop, M.R. and MINH, N., 2011. Characteristics of the London Clay from the Terminal 5 site at Heathrow Airport. In Stiff Sedimentary Clays: Genesis and Engineering Behaviour: Géotechnique Symposium in Print 2007 (pp. 167-182). Thomas Telford Ltd.

- Carder D R (1995). Ground movements caused by different embedded retaining wall construction techniques. TRL Report TRL172. Transport Research Laboratory, Crowthorne.

We thank the reviewers again for these well-informed and helpful comments.