Title: Bayesian optimisation for CPT-based prediction of impact pile driveability

Article number: JRNGTENG-SS-22-00899

We are grateful to the AE and reviewers for their time in reviewing the manuscript and for their useful and insightful feedback. We have responded to the queries individually below and modified the manuscript as appropriate. The changes to the manuscript are highlighted in yellow.

# Associate Editor

The paper investigates mainly the use of Bayesian optimization of the coefficients in the Lehane et. al. 2020 unified design equations to improve pile driveability predictions, particularly for piles with low slenderness ratios, as is the case for monopiles. There are some questions regarding the selection made of a single COV value, and the selection of deterministic values for quake and damping (these depend on soil type, and this assumption needs to be further considered given that the authors acknowledge their significant impact on predictions). It would also be important to discuss why the predictions with the coefficients proposed by Lehane et. al. 2020 are close to the measured data (is the method predicting end of driving resistances and not at some time later? or is this related to other factors?). It would be good to expand the role of pile diameter on various aspects of the formulations (e.g., on the degradation term) and where else would it make sense to have it appearing, with a clear connection to physical phenomena. Why use qt' and not qc since the soils are sandy soils? Finally, I suggest that all the input data from the case studies that were used in the paper be available for others trying to use IMPACT to reproduce the results presented in this paper.

Response: Thank you, we have clarified some of these points in the response to the reviewers. We have added responses on your additional queries below.

- The Lehane et al. (2020) method is intended to represent resistance available at around 14 days after driving and may therefore may not be representative of SRD where changes in capacity with time occur after driving. Such changes are likely to occur along the shaft only and be relatively small in sandy soils. While the match with the observed data is reasonable with the unified sand method, it is not perfect which likely represents the time period for which the method is calibrated. We have added additional clarification in the text on this point.

- The diameter term could also be included in the base resistance formulation such as been suggested by others previously (Chow, 1997, Jardine et al., 2023). We have added additional clarification in the text on this point.

# Reviewer 1

The paper presents a study on improving the quality of (CPT-based) pile drivability predictions using Bayesian optimisation and pile installation data for identifying best parameters for impact pile hammering. The study compares the results of the proposed method to those returned by methods already available in the literature (and used in practice), with the new approach demonstrated using a case study from a German offshore wind site. As existing drivability models are increasingly challenging the performance of available predictions methods (particularly due to the growing diameter of offshore monopiles), the topic of the paper is very topical. The proposed work-flow seems to positively impact the state of practice, though without remedying the fundamental limitations of existing drivability models.

Given the strong need for more work along this line of research, I am in favour of publishing this contribution. A few suggestions for improvement are provided below, along with a number of minor comments in the annotated manuscript.

- The papers gives overall the impression that discrepancies between drivability predictions and observations for larger-diameter monopiles are mostly due to inaccurate SRD. While that aspect is certainly there, I believe it should be more openly recognised that usual 1D stress wave theories (i.e., treating a monopile as a 1D bar) are simply inaccurate for cylindrical shells of larger and larger diameter. For instance, with increasing diameter, pile vibrations produce several lateral expansion/contraction due to Poisson's effect (see Tsetas et al. (2021)) so that the input energy is also "consumed" along the radial direction. Neglecting such effect may lead to "perceiving" an increased SRD, which is a typical (though not widely recognised) drawback of standard approaches. It is worth mentioning that the Authors reach a related conclusion when commenting on the incomplete account of L/D effects in current methodologies. It would be appropriate to take the chance to inform the readership about fundamental gaps that overlap with the mentioned uncertainties in parameter identification.

Response: Thank you. We appreciate this point and have added the following to the text.

“Pile driveability predictions in this study were preformed using industry-standard 1D wave equation analyses. Tsetas et al. (2021) compared 1D models with a 3D model that accounted for wave dispersion and non-local soil reaction and showed that the 1D models become less accurate for diameters greater than ≈4m, above the maximum D considered in this study.” (pg 5)

“Additional considerations related to the modelling approach may also be required for larger diameter piles; see Tsetas et al. (2021).” (pg 17 conclusions)

- Lines 128-136: The quake u_q and damping parameter J (both for shaft and tip) are regarded as empirical parameters, in addition to those in Eq. (6). It would be worth commenting on the validity of the overall approach, given that these parameters have been mostly established after the analysis of small-diameter pile data, with loose connection to physical soil properties. More "rational" soil resistance models have been obtained by introducing spring and dashpot elements with parameters directly associated with the impedance functions of solid piles. Recently, Kaynia et al. (2022) proposed a model that uses impedance functions suitable for tubular piles, with a focus on vibratory driving (note that its application to impact driving is not precluded). Please consider including considerations (and references) that inform about going beyond 1D stress-wave theories. It is a belief of this reviewer that improving the underlying physics of drivability models is a more urgent (and cheaper) endeavour than proceeding to additional (and expensive) field testing exercises - which would otherwise continue to be interpreted via an inadequate theory.

Response: thank you, we have added additional references to the text to more complex models, and also some explanation on the choice to use the simplified Smith model. We have also added an additional sentence to the conclusions.

Several soil rheological models of varying complexity have been developed to model the response during driving (see e.g. Simons and Randolph, 1985, Deeks and Randolph, 1993, Salgado et al., 2015, Kaynia et al., 2022). While IMPACT includes several of these advanced options to model the soil and base resistance, the pile driving soil base and shaft resistances in this study were modelled in this study using the Smith (1962) approach. While the limitations of the Smith approach are well documented (see e.g. Buckley et al., 2017) it remains the most commonly employed tool by industry for simple and practical driveability analysis. (pg 7)

Application of the proposed framework to a larger database of pile driving records with a greater range of L/D ratios is warranted to further investigate the trends observed as is consideration of additional, more advanced, rheological models whose parameters could be included in the vector of uncertain variables given in Eq. 8.

- Regarding the quake and damping coefficients, would it be possible to comment on the possibility to incorporate the latter into the vector of uncertain variables \theta in Eq. (8)? As these empirical parameters may also belong to the set of uncertain variables, it would not be irrational to argue that their range of values could also vary as large-diameter piles are considered. The findings of this paper seem to support such a statement.

Response: Thank you. We have included the below in the text. Please also see our response to comment 1.

Fixed values have been adopted for the Smith parameters in this study (i) for simplicity and to demonstrate the optimisation framework and (ii) to compare with predictions made with the conventional SRD approach. However, we accept that these parameters may also vary with soil type and pile geometry and thus be affected by the limitations of the method. A brief statement to this effect has been included in the revised manuscript (pg7/8)

- Following up on comment (3): should those parameters be included in the optimisation process, would it be possible to consid8er them as statistically independent from the rest of the variables? It is known that the search of suitable soil resistance, quake and damping coefficient may admit multiple solutions (non-uniqueness), which suggests that the mentioned parameters are likely inter-dependent. How would considering such inter-dependence affect - or even hinder - the proposed optimisation procedure?

Response:

The reviewer is correct that such an optimisation process (of an undetermined system) has multiple non-unique solutions. In the present study, we assume the uncertain SRD parameters to be updated are independent as each model parameter plays a different role and has a different practical meaning. Whilst achieving inter-dependence is certainly possible by considering multivariate statistical distributions, this leads to a notable increase in the complexity of the optimisation problem, and therefore the computational cost. The proposed approach was developed to target an optimal balance between computational efficiency, and thus likelihood for industry take-up, and statistical rigor. Additional commentary on this issue is included in the revised paper (lines X – X).

# Reviewer 2

General evaluation

I’ve found this a generally well written, interesting and original contribution to a subject of relevance for the journal readership. There are a number of aspects that in my opinion require clarification and/or correction before publication. My comments below are oriented to help in this respect.

Observations

- What is the rationale for the initial assumption COV = 0.3 for all parameters? Does the value assumed affect the optimization output or algorithm performance? Please comment

Response: The reviewer raises an important point which was perhaps under-stated in our previous submission. We adopted COV = 0.3 for all parameters to achieve ‘weak’ priors such that the posterior distributions are largely dictated by the evidence, specifically the likelihood distributions of the monitored data. Conversely, a low COV indicates strong belief in the adopted prior parameters such that posteriors are more heavily influenced by our prior estimates. With access to large databases of monitored data, the authors expect that future research will be able to make use of COV values which are tailored to the relative confidence in a given parameter. A short note on this point has been included in our revised submission (lines X – X).

- L132-133 I am puzzled about this choice: quake and damping are famously obscure parameters, and you recognize their large influence in output... why were they excluded from the optimization? Is this a fundamental limitation of the method? Some explanation seems necessary

Response: Thank you. We have included the below in the text. Please also see our response to comment 1.

Fixed values have been adopted for the Smith parameters in this study (i) for simplicity and to demonstrate the optimisation framework and (ii) to compare with predictions made with the conventional SRD approach. However, it is accepted that these parameters may also vary with soil type and pile geometry and thus be affected by the limitations of the method. (pg 7/8)

Application of the proposed framework to a larger database of pile driving records with a greater range of L/D ratios is warranted to further investigate the trends observed as is consideration of additional, more advanced, rheological models whose parameters could be included in the vector of uncertain variables given in Eq. 8. (pg 17, conclusions)

- The reference given for IMPACT is not useful, as it cannot be found easily. Is there not something more accessible?

Response: Thank you, we have added an additional reference to Randolph (1990). IMPACT is now available as a scalable web-app via geocalcs.com. We have also added this to the text (pg 7)

- L99 Where are the pore pressure peaks of up to 1MPa? they do not seem visible in the CPTu traces

Response: Thank you we have removed this statement as it does not relate to the locations considered (pg 6).

- L116 Using different advances per blow makes the comparison between the two differently sized piles complicated. Maybe you can comment on the total driving energy per m (work done on the pile).

Response:

- I think you have not defined D_i in Equation 3

Response: Thank you we have added this

- L181-L188 “parameter” is a tricky word in this paragraph, because the stochastic variables in Θ are “parameters” of your soil resistance model but also the joint PDF used to describe them includes “parameters” (mean and variance, as they are assumed uncorrelated). Please clarify the text.

Response: Thank you, we have clarified this in the text.

- You should define the acronym SMC

Response: Thank you, we have clarified this in the text.

- Reference to Ching & Chen (2007) is missing from the list

Response: THank you, we have corrected this.

- You only refer to Bayesian applications in geotechnical problems (excavations, embankments…) that seem somewhat distant to the one addressed here. As a closer precedent, you might like also to mention the recent work on offshore dynamic CPTu by Collico et al. (2022)

Response: Thank you we have added this reference

- It is unclear if you are modifying someone else sampling algorithm or not. If the second, I wonder how helpful for the reader is the brief description that you introduce and if cannot be just replaced by appropriate references.

Response: The authors do not develop a bespoke sampling strategy for this study and the reviewer makes a valid point regarding the level of detail included. While we have stopped short of providing an exhaustive description, we felt that the detail currently in the manuscript is important to ensure the paper is standalone. There is also considerable precedent in this journal for providing detail on the sampling strategy given that it has an important influence on the optimisation efficiency and results.

- L220 A “Gaussian proposal” for what?

Response: The authors thank the reviewer for spotting this. We refer to a Gaussian proposal PDF used to generate Markov chain sample. This has been corrected.

- Figure 7 suggests that you optimize parameters by adding blow results sequentially to the optimization i.e. that you optimize initially using only one blow (i = 1) , then using results for two blows (i = 2) , etc. That seems strange, as you will give much more weight in the optimization to the first blows. Please clarify.

Response: Thank you, we have clarified the figure

- L238 What are “unseen calibrations”? In figure 8 you talk about “unseen predictions”,

Response: Thank you. This is a typographical error and has been corrected.

# Annotated PDF

- L25  but also effectively

Response: Thank you we have corrected this

- L29 it may also be "physics-based"

Response: Thank you we have added this

- L43 Corrected?

Response: Thank you we have added this

- L45 give this one as Eq. (3)

Response: Thank you we have added this

- L50 typo

Response: Thank you we have corrected this

- L130 nodal velocity?

Response: Thank you we have added this

- L131 is this a wrong equation reference?

Response: Thank you we have corrected this

- L157 (or unplugged)

Response: Thank you we have added this

- L162 there is no Eq. (7)

Response: Thank you we have corrected this

- L170 add citations in support of this statement

Response: Thank you we have added this

- L180 - also referred to as conditional probability

Response: Thank you we have added this

- L365 It would be good to add a closing paragraph - it seems a bit odd to conclude with a bullet point.

Response: Thank you we have edited this

- Figure 6 if the slider is plastic, then the spring is elastic and the dashpot viscous.

Response: Thank you we have corrected this figure

- Barbosa et al please check if JGGE allows citation of personal communications

Response:

- References: integrate conference paper citations with all the necessary details - editor of the proceedings, page numbers, location, date of the conference, etc. Most conference paper citations are incomplete.

Response:

# References

Buckley, R. M., Kontoe, S., Jardine, R. J., Maron, M., Schroeder, F. C. & Barbosa, P. 2017. Common pitfalls of pile driving resistance analysis - A case study of the Wikinger offshore windfarm. Proc. 8th Intl. Conf. Offshore Site Investigation and Geotechnics. London, UK: Society of Underwater Technology.

Chow, F. C. 1997. Investigations into Displacement Pile Behaviour for Offshore Foundations. PhD, Imperial College London.

Deeks, A. & Randolph, M. 1993. Analytical modelling of hammer impact for pile driving. International Journal for Numerical and Analytical Methods in Geomechanics, 17, 279-302.

Jardine, R. J., Buckley, R. M., Liu, T. F., Byrne, B. W., Kontoe, S., J.;, Mcadam, R. A., Schranz, F. & Vinck, K. 2023. The axial behaviour of piles driven in chalk - Géotechnique -  in press.

Kaynia, A. M., Hebig, J., Pein, T. & Shin, Y. 2022. Numerical model for dynamic installation of large diameter monopiles. Soil Dynamics and Earthquake Engineering, 161, 107393.

Lehane, B., Liu, Z., Bittar, E., Nadim, F., Lacasse, S., Jardine, R., Carotenuto, P., Jeanjean, P., Rattley, M. & Gavin, K. A new 'unified' CPT-based axial pile capacity design method for driven piles in sand.  4th International Symposium on Frontiers in Offshore Geotechnics, 2020. American Society of Civil Engineers, 463-477.

Randolph, M. F. 1990. Analysis of the dynamics of pile driving. Developments in Soil Mechanics IV: Advanced Geotechnical Analyses. Elsevier.

Salgado, R., Loukidis, D., Abou-Jaoude, G. & Zhang, Y. 2015. The role of soil stiffness non-linearity in 1D pile driving simulations. Géotechnique, 65, 169-187.

Simons, H. A. & Randolph, M. F. 1985. A new approach to one dimensional pile driving analysis. Intl. Conf. on Numerical Methods in Geomechanics. Nagoya, Japan.

Smith, E. a. L. 1962. Pile-driving analysis by the wave equation. J. Soil Mech Found. Eng. - ASCE, 4, 35-61.

Tsetas, A., Tsouvalas, A. & Metrikine, A. V. 2021. Installation of large-diameter monopiles: Introducing wave dispersion and non-local soil reaction. Journal of Marine Science and Engineering, 9, 313.