Machine Learning Assisted Constitutive Modelling of Stress-induced Anisotropic Behaviour of Clay

** January 2023

The authors gratefully acknowledge the additional constructive comments from the Assessor and Reviewers. The revised manuscript has been modified to address all comments, and are described point-by-point below. Changes in the revised manuscript are highlighted in yellow for convenience.

Reviewer #1

Comment 1: I found the paper to be overall reasonable, with reasonable logic, and a reasonable outcome. I found the discussion under the model's robustness evaluation a bit unnecessary - the translation that I perceive is: "other models do badly outside of parameter space and ours does too but just as bad." I am sure that the authors will fix the message there if indeed this is not quite what they meant.

Reply:

Generalization outside the original training space is a key pain point associated with the use of machine learning techniques for practical applications. However, our work shows that the proposed methodology can indeed predict outside the calibration space with reasonable accuracy and reliability. We believe the associated discussion on model robustness is essential for practicing engineers to reliably take up the developed techniques. However, we take the reviewer’s point that the message in our previous submission required refinement; we have edited these points in lines 440-452 in the revised paper.

Comment 2: The only thing missing from the paper is an extra schematic of with a conceptual flow chart of the work process.

Reply:

This is a sensible suggestion. We have added a new Fig. 2 to introduce the flow chart of the proposed methodology.

Reviewer #2

Comment 1: Line 32: Omit "the typically" and replace with "a"

Reply:

Thanks for your suggestion, it has been changed.

Comment 2: Line 33: Add a comma after "parameter"

Reply:

This line has been corrected.

Comment 3: Lines 37 - 38: What do you mean by "learn" critical state soil mechanics? is it learning actual mechanics theory or is it just learning the model's description of critical state behavior which is influenced by the model assumptions? Does it somehow estimate functional form or derive physics-based interpretations?

Reply:

The reviewer raises a valid point and, in retrospect, the authors should have been more specific on this point. The machine learning algorithm learns the model’s description of critical state behaviour. This description is indeed influenced by model assumptions, which is why the authors also investigated the sensitivity of the MRNN performance to the adopted constitutive model and parameters (see “Influence of base constitutive model” section). MRNN therefore incorporates these physics-based interpretations by training on the data generated by the critical state soil models. We have rephrased these sentences to make it clearer (lines 12-13, 151-154 and 197-206).

Comment 4: Regarding the statement "the model is also shown to be insensitive to the exact composition of the synthetic…" what was the justification for this assessment?

Reply:

This statement is also based on the results presented in the section “Influence of base constitutive model” and specifically Fig. 13. The results show that the final performance of the MRNN is similar, regardless of the underpinning constitutive model or parameter set using for the first phase of training.

Comment 5: Lines 125 - 126: How do you verify that it is identifying the appropriate cause-effect relationship?

Reply:

This judgement is based on the model performance on the unseen ‘testing’ data and quantified using indicators such as the mean squared error and mean absolute percentage error, as is typical in the machine learning field. We have clarified this sentence, as shown in lines 101-102.

Comment 6: Lines 143 - 146: But if the dataset is not large enough then the high-fidelity model will be "off" so comparing it to the low fidelity will also be "off" if used as a calibration reference. So how is this addressed? DO you gather a large amount of data to start with? if so what is the threshold that determines a "sufficiently large" dataset?

Reply:

This is an important point. Any constitutive model requires a certain amount of data for validation which is inevitably dependent on the model itself. Our research has shown that MCC-MR-NN can be well calibrated and outperforms conventional constitutive models (e.g. modified Cam clay) using the same high-fidelity dataset size. Conventional constitutive models generally can be well calibrated using 3 sets of experiments with different confining pressures. Therefore, the proposed model is compatible with existing model calibration procedures and data requirements thus ensuring its practical applicability. A short note on this point has been included in the revised manuscript (lines 143-145).

Comment 7: Line 154: What is the "rigorous" mathematical framework? Also replace "though" with "through"

Reply:

The mathematic framework is described in Eqs. 1-5 and this has now been clarified. “though” has also been replaced with “through”.

Comment 8: Lines 156 - 157: What is this "more advanced anisotropic bounding surface model"? and how does it differ in features and components to the base model used in this study?

Reply:

The description of the advanced bounding surface model was given in section “Anisotropic bounding surface model”. It differs to the base (MCC) constitutive model in the shape of the yield surface and the hardening law and plastic flow. Additional details are provided in the sections “Modified Cam-Clay model” and “Anisotropic bounding surface model”.

Comment 9: Should the σ_(L,p)^(t+) actually be σ_(E,a)^(t+)?

Reply:

We are not sure what line or equation of the manuscript the reviewer is referring. However, we have re-checked all notation and do not see any typographical errors.

Comment 10: Lines 210 - 214 (and overall manuscript): The description of the model framework and development is not clear. A lot of jargon and acronyms are used describe how the model is built and its components, however it is not clear how each component adds to the model and overall, how the model estimates simple stress strain behavior (i.e., is it stress-based or strain-based? The MCC takes in strain increments and estimates the stresses using critical state parameters M, eta, confinement, initial void ratio, hardening factor, etc. How are those incorporated here? The mentions of "learning" as in soil mechanics does not relay any detail other than giving the reader an impression of a black box step that cannot be fully substantiated. The use of machine learning to conduct constitutive model development is nice but not if one does not have intricate knowledge of what is going on under the hood and how that relates to/impacts the modeling assumptions and outputs. Essentially, what is going in and what is coming out and how do those two relate to geotechnical behavior of soils?

Reply:

The authors thank the reviewer for this constructive comment. We have added a new Fig. 2 to better describe the proposed model framework together with the text presented in the section “MCC-MR-NN for constitutive modelling”. Actually, constitutive modelling using the proposed MCC-MR-NN has the same inputs as the conventional stress integration algorithms.

We have also added a pseudocode (Algorithm 1) to introduce the process of using the proposed methodology to predict stress-strain relationship. There are two key processes by which the proposed modelling captures the geotechnical behaviour of soils:

- The model is first trained on a database of simulations generated by an adopted constitutive model. This is the “low-fidelity” model. By training on this synthetic dataset, it implicitly learns the constitutive model’s description of geotechnical behaviour to encode high-level cause-effect relationships.

- The model is then refined by comparing predictions from step (1) above to real experimental data and incorporating these differences into the “high-fidelity” model; its form is learned directly from the geotechnical data.

T

hese points was added to lines 197-206 in the revised paper.

Comment 11: Line 217: Does "" mean that it could estimate it is still plausible for ML-assisted models to generate unexpected predictions behaviors that defy physical constraints and limits (i.e., unrealistic behaviors)?

Reply:

Exactly. This also includes predictions exhibiting large errors. We have emphasized these two points in line 211 of the revised submission.

Comment 12: Line 219: What is dropout- based uncertainty? how does it relate to unrealistic estimates or better yet mitigating them?

Reply:

We have added a new Fig. 3 to explain dropout-based uncertainty. Essentially, this involves initialising the same neural network many times and for each initialisation, each neural connection is ‘cut’ with a probability of p. This results is an estimate of the sensitivity of model predictions to the exact model architecture (e.g. by quantifying the mean and standard deviation of the output predictions) and therefore an estimate of epistemic (model) uncertainty. This process is described by equation (6), where r equals either 0 or 1 following a Bernoulli distribution with probability p. Additional text has been added to the paper for clarification (lines 213 – 223).

Comment 13: Line 226: What does a dropout probability of 0.1 mean?

Reply:

Building on the description above, a dropout probability of p = 0.1 means 10% of all neuron connections are cut and do not contribute to the computation. This has now been defined in lines 225-226 in the revised paper.

Comment 14: Line 227: What is "r" and why is it important to the dropout? why does it follow a Bernoulli distribution? What does it do to the output of equation 6? Why is equation 6 needed? What does it represent?

Reply:

For a given neural connection, the parameter r determines whether the connection is to be cut by adopting only a value of either 0 (cut) or 1 (no cut) in equation (6). This mathematical process is achieved using a Bernoulli distribution. As mentioned in our response above, this is to achieve many slightly different initialisations of the same neural network architecture to quantify the sensitivity of the predictions to the exact model architecture. This is the classical uncertainty based approach for neural networks in the machine learning literature.

yl+1=fwl+1yl+bl+1

yl+1=fbl+1

Comment 15: Lines 295 - 296: what was the prior resolution? (i.e., how many data points per loading path before upsampling?)

Reply:

The prior resolution was variable, as is the case with any experimental data. The number of datapoints in each loading path ranged between 10 to 100. We used interpolation to standardise the number of datapoints for the purposes of comparison. This point has been explained in lines 292-294 of the revised paper.

Comment 16: Line 315 - 321: How did the NN_L predict this? what parameters were given to the NN_L model? The fact that the synthetic data and the NN_L predictions are essentially identical do you think this may be due to overfitting? If not, what is your explanation for this and how do you justify it as the proposed approach is working versus not being sure what is really going on with regards to the ML model NN_L?

Reply:

The introduction of NNL together with its input parameters are now given in pseudocode ‘Algorithm 1’. The excellent performance on the unseen testing dataset confirms that overfitting was not encountered in this study. It is worth noting that Fig. 4 shows the predictions of NNL on the MCC-generated synthetic dataset only; these comparisons demonstrate that NNL can accurately reproduce the MCC results. This process does not involve the experimental data thus explaining the very strong agreement observed in these figures. The authors clarify this point in lines 315-319 in the revised paper.

Comment 17: Line 335: Why was there no drained test for CK_0?

Reply:

Fig. 8 shows the drained tests for CK_0. We have made this clear in the revised paper (lines 370-371).

Comment 18: Lines 340 - 342: Was the model performance evaluated using another soil source (i.e., another clay of till)?

Reply:

Due to the word count limit for this journal, only experimental data for LCT clay were used in this study. The model performance was evaluated using this clay with different loading paths and stress histories, as described in the section “Data generation, preprocessing and verification”.

Comment 19: Lines 342 - 343: I suggest substantiating this claim quantitatively (error analysis) rather than qualitatively (i.e., "excellent agreement")?

Reply:

Quantitative analysis was indeed provided in the original manuscript (see Table 4).

Comment 20: Lines 348 - 350: the values in this table should be explained to give context.

Reply:

The definitions of these indicators are provided in lines 322-327 in the revised manuscript.

Comment 21: Lines 356 - 358: Is this referring to the fact that the stress strain curve crosses η = 1.2 at small strains?

Reply:

We have clarified the reason in lines 362-366 of the revised paper.

Comment 22: Line 360: What is the K0 value being used in for all the experimental data in the plots?

Reply:

All K0 tests correspond to a value of K0 = 0.5. All details were summarized in Table 2.

Comment 23: Line 367: From figure 5b it seems like the trend goes past -0.86 for more negative strains. What is the cause of this?

Reply:

Me = -0.86 was directly extracted from the cited paper (Gens, 1982), which was estimated based on all data rather than only Fig. 7 (original Fig. 5). Therefore, it is natural that some experimental data slightly exceeded the critical state line with the slope of -0.86. However, they key takeaway is that the predictions matched well with the experimental data.

Comment 24: Lines 374 - 375: Are you saying that the ANN model is not founded or constrained by geotechnical principles/behaviors and physics?

Reply:

We thank the reviewer for spotting this typographical error. The low-fidelity model was weakly constrained by physics (by training on data generated by an existing constitutive model) and the high-fidelity model was used to refine these results in light of the experimental data (without constraints). The corresponding lines in the manuscript have been corrected (lines 197-206 and 383-385 in the revised paper).

Comment 25: Line 386: Replace "is possible" with "is appropriate"

Reply:

The authors are grateful for this suggestion, it has been changed.

Comment 26: Lines 418 - 420: Doesn't this indicate that it may be possible that the basic model may be overfitting or that the advanced model may have some error? Or that the advanced model actually is not an advanced model but an augmented basic model?

Reply:

Actually, it can be seen from Fig. 13 that MR-NN incorporating the advanced model results in a slightly improved mean accuracy and a decrease in the maximum error. Nevertheless, it can be concluded that predictions obtained using a basic and more advanced soil model yield broadly similar results, as mentioned in lines 428-430.

Comment 27: Lines 424 - 425: Was an exercise to support this claim undertaken with other soil data or data not used in the current investigation?

Reply:

Application of the proposed methodology to other soils is outside the scope of this paper given the word count limits for this journal. However, recent work we recently applied a similar methodology to model rate-dependent behaviours of Hong Kong Marine deposits and Merville clay (He et al., 2022). The results also verify the robustness of the proposed methodology.

Comment 28: Line 431: Replace "is one" with "and is one"

Reply:

This has been changed.

Comment 29: Lines 432 - 435: I am confused as to how this downsampled and upsampled dataset deviates significantly from the original dataset? Is it not still the same dataset being used? also are the upsampled and downsampled lines in fig 12 a&b new regressed ML models? I am not sure how this supports robustness when using the same data.

Reply:

In hindsight, this point should have been made more clear in our original submission. The reviewer is correct that it is the same loading path. However, the strain increments in each case are very different which can often lead to computational and predictability problems. Therefore, upsampling and downsampling the same loading path allowed for comparisons of model performance for different strain increments. We have clarified this point in lines 444-445 in the revised paper.

Comment 30: Line 437 - 438: where is this "large initial strain increment" on Figure 12?

Reply:

Please refer to our response to comment 29.

Comment 31: Line 448: Insert "that" before "the"

Reply:

Thank you for your suggestion, it has been added.

References:

Gens, A. (1982) Stress–strain and strength of a low plasticity clay.)  University of London, London, UK, vol. Ph.D.

He, G.-F., Zhang, P., Yin, Z.-Y., Jin, Y.-F. & Yang, Y. (2022) Multi-fidelity data-driven modelling of rate-dependent behaviour of soft clays. Georisk: Assessment and Management of Risk for Engineered Systems and Geohazards:1-13.